RISC-V AI SoC

Complete Project Blueprint & Execution Roadmap

For VLSI + AI Fresher Job Placement

What You Are Building One unified SoC built across 5 phases — not 5 separate projects. RISC-V RV32I Core → L1 Cache → AXI4 Interconnect + APB Bridge → AI Accelerator (MAC Array) → Full System Verification Each phase adds one layer to the same chip. The final result is a complete, synthesized, formally-verified AI SoC that you present as ONE flagship project on your resume and in every interview.

# Open-Source Tool Stack

| RTL Writing | SystemVerilog — VS Code + Verilog-HDL extension |
| --- | --- |
| Linting | Verilator --lint-only (catches 90% of bugs before simulation) |
| Simulation | Icarus Verilog (iverilog + vvp) + GTKWave for waveforms |
| Verification | cocotb — Python-based testbenches (modern, industry-adopted) |
| Formal Verification | SymbiYosys (sby) — replaces JasperGold for your arbiter proofs |
| Synthesis | Yosys — RTL to netlist |
| Timing Analysis | OpenSTA — replaces Synopsys PrimeTime |
| Place & Route | OpenROAD — full P&R flow |
| Full Flow | OpenLane — wraps Yosys + OpenROAD, gives GDSII + reports |
| PDK | SkyWater Sky130 — Google's open 130nm PDK, used in real tapeouts |
| Power Reports | OpenROAD power analysis — real mW numbers for resume |

Installation (Ubuntu — TheBeast) sudo apt install iverilog gtkwave verilator yosys symbiyosys pip install cocotb openlane # OpenLane downloads Sky130 PDK automatically on first run

| Phase 0 Repository & Environment Setup | ⏱ Week 1 — Days 1-2 |
| --- | --- |

Before writing a single line of RTL, set up the GitHub repository and folder structure that will house all 5 phases. This structure is what recruiters and interviewers will browse — make it clean.

### GitHub Repository Structure

Folder Layout riscv-ai-soc/ ├── README.md ← Your project story (write this last) ├── docs/ ← Architecture diagrams, reports ├── phase1_riscv_core/ │ ├── rtl/ ← All .sv source files │ ├── tb/ ← cocotb testbenches │ ├── formal/ ← SymbiYosys scripts │ └── synth/ ← Yosys scripts + reports ├── phase2_cache/ ├── phase3_axi_interconnect/ ├── phase4_ai_accelerator/ ├── phase5_integration/ ← Full SoC integration └── openlane/ ← OpenLane config for final synthesis

### Phase 0 Checklist

| ☐ | Create GitHub repo: riscv-ai-soc (public) |
| --- | --- |
| ☐ | Install all tools: iverilog, gtkwave, verilator, yosys, cocotb, openlane |
| ☐ | Verify iverilog: iverilog -V ¦ Verify cocotb: python -c 'import cocotb' |
| ☐ | Create folder structure as shown above |
| ☐ | Add .gitignore (ignore simulation dumps, waveform files, build artifacts) |
| ☐ | Write a placeholder README.md — you'll fill this in at the end |
| ☐ | Commit and push initial structure |

| Phase 1 5-Stage RISC-V RV32I Pipeline Core | ⏱ Weeks 1-3 |
| --- | --- |

You already have this project. Phase 1 is about cleaning it up, re-verifying it with cocotb instead of your old UVM testbench, running it through Yosys, and documenting it properly so it fits the unified SoC story.

### What You Have vs What Needs to Change

| 5-stage pipeline RTL | Keep as-is — already solid |
| --- | --- |
| Hazard detection / forwarding | Keep — verify completeness with cocotb |
| Branch prediction | Keep — document the strategy (static taken/not-taken) |
| UVM testbench | Replace with cocotb — simpler, more portable, still impressive |
| Synopsys DC synthesis | Replace with Yosys + OpenSTA — same output, open-source |
| AXI interface | ADD THIS — expose instruction and data memory ports as AXI4-Lite masters |

### Key RTL Modules

| fetch_stage.sv | PC register, instruction memory interface, branch mux |
| --- | --- |
| decode_stage.sv | Instruction decode, register file read, control signals |
| execute_stage.sv | ALU, branch resolution, forwarding mux |
| memory_stage.sv | Data memory interface (will connect to cache in Phase 2) |
| writeback_stage.sv | Register file write, result mux |
| hazard_unit.sv | Stall and flush control logic |
| forwarding_unit.sv | Data forwarding path mux selects |
| riscv_core.sv | Top-level that wires all stages together |

### cocotb Verification Plan

- Test 1 — Arithmetic: add, sub, and, or, xor, sll, srl, sra, slt, sltu

- Test 2 — Load/Store: lw, sw, lb, sb — check memory roundtrip

- Test 3 — Branch: beq, bne, blt, bge — verify taken and not-taken paths

- Test 4 — Hazard: RAW data hazard sequences — verify forwarding corrects result

- Test 5 — Full program: Bubble sort on 8-element array — end-to-end correctness

### Yosys Synthesis

Yosys Script (synth/synth.ys) read_verilog -sv rtl/riscv_core.sv synth -top riscv_core dfflibmap -liberty sky130_fd_sc_hd__tt_025C_1v80.lib abc -liberty sky130_fd_sc_hd__tt_025C_1v80.lib stat # Prints area in cells write_verilog synth/riscv_core_netlist.v

### Phase 1 Checklist

| ☐ | RTL compiles clean with: verilator --lint-only riscv_core.sv |
| --- | --- |
| ☐ | All 5 cocotb tests pass with zero assertion failures |
| ☐ | Waveform captured in GTKWave for at least one test — screenshot for docs/ |
| ☐ | Yosys synthesis completes — note cell count and critical path delay |
| ☐ | Commit to phase1_riscv_core/ with clear commit messages |
| ☐ | Add AXI4-Lite master ports to memory_stage.sv (prep for Phase 3) |

Resume Bullet for Phase 1 Designed 5-stage pipelined RV32I processor in SystemVerilog with full hazard detection, data forwarding, and branch resolution. Verified with cocotb (5 test suites, bubble sort end-to-end). Synthesized with Yosys on Sky130 PDK.

| Phase 2 Direct-Mapped Write-Back L1 Cache Controller | ⏱ Weeks 3-4 |
| --- | --- |

You already have this too. Phase 2 is about integrating the cache with the RISC-V core's memory port, upgrading your Python golden model to a proper cocotb test, and running a hit/miss analysis that produces a table you can show in interviews.

### Key RTL Modules

| cache_controller.sv | FSM: idle → hit → miss → fill → writeback states |
| --- | --- |
| cache_data_array.sv | SRAM-style array: 64 sets, 1 way, 4-word blocks (256-bit) |
| cache_tag_array.sv | Tag + valid + dirty bit storage |
| cache_top.sv | Top-level: CPU-side AXI4-Lite slave + memory-side AXI4 master |

### Integration with RISC-V Core

The cache sits between the RISC-V data memory port and main memory. The CPU sends AXI4-Lite read/write requests to the cache. On a hit, the cache responds in 1 cycle. On a miss, it fetches from main memory via a full AXI4 burst and then responds.

### cocotb Verification Plan

- Test 1 — Sequential read hits: fill cache then re-read same addresses

- Test 2 — Cold miss then hit: first access always misses, second always hits

- Test 3 — Dirty eviction: write to address A, then force eviction by accessing conflicting set — verify writeback occurs

- Test 4 — Stress: 1000 randomised read/write accesses compared against Python golden model

- Coverage metric: report hit rate and miss rate — target >80% hit rate under sequential workload

### Hit/Miss Analysis Table — What to Produce

| Access Pattern | Hit Rate |
| --- | --- |
| Sequential (stride-1) | ~95% |
| Strided (stride-8) | ~60% |
| Random | ~30% |
| Repeated loop body | ~98% |

Generate this table with cocotb by running four access pattern workloads and reporting the hit counter. This is your interview talking point — 'I measured cache behaviour across four access patterns.'

### Phase 2 Checklist

| ☐ | Cache integrates with RISC-V core's data memory AXI4-Lite port |
| --- | --- |
| ☐ | All 4 cocotb tests pass — zero golden model mismatches |
| ☐ | Hit/miss table generated and saved to docs/cache_analysis.md |
| ☐ | Dirty eviction waveform captured in GTKWave — screenshot saved |
| ☐ | Yosys synthesis of cache_top.sv — note area |
| ☐ | Commit to phase2_cache/ folder |

Resume Bullet for Phase 2 Implemented direct-mapped write-back L1 cache (64 sets, 4-word blocks) integrated with RISC-V core via AXI4-Lite. Verified with cocotb against Python golden model. Measured hit rate across 4 access patterns; achieved 95% hit rate under sequential workload.

| Phase 3 AXI4 Crossbar Interconnect + AXI-APB Bridge | ⏱ Weeks 4-5 |
| --- | --- |

You have this project too. Phase 3 is about wiring everything together — the RISC-V core, cache, and future accelerator all communicate through this interconnect. You also need to add formal verification of the round-robin arbiter using SymbiYosys.

### System Role of the Interconnect

The AXI4 crossbar is the backbone of your SoC. It routes transactions between:

- Master 0: RISC-V core instruction fetch port

- Master 1: RISC-V core data memory port (through cache)

- Master 2: AI Accelerator DMA port (added in Phase 4)

- Slave 0: Main memory (SRAM model)

- Slave 1: APB peripheral space (via AXI-APB bridge)

- Slave 2: AI Accelerator register map (memory-mapped control)

### Key RTL Modules

| axi4_crossbar.sv | 3-master 3-slave crossbar, round-robin arbitration per slave |
| --- | --- |
| axi4_arbiter.sv | Round-robin arbiter — this is the one formally verified |
| axi4_apb_bridge.sv | Protocol converter: AXI4 to APB, handles bursts and back-pressure |
| cdc_sync.sv | 2-FF synchroniser for any cross-domain signals |
| sram_model.sv | Behavioural SRAM — 64KB, AXI4 slave interface |

### Formal Verification with SymbiYosys

Your JasperGold arbiter verification is replaced 1-for-1 with SymbiYosys. Same SVA properties, same proof goals — just open-source tooling.

SymbiYosys Script (formal/arbiter.sby) [options] mode prove depth 20 [engines] smtbmc boolector [script] read -formal axi4_arbiter.sv prep -top axi4_arbiter [files] rtl/axi4_arbiter.sv

### SVA Properties to Prove

- Mutual exclusion: no two masters granted same slave simultaneously

- Fairness: every pending master receives a grant within N cycles (liveness)

- No starvation: a continuously requesting master is never permanently denied

- Grant stability: once granted, grant held until transaction completes

### Phase 3 Checklist

| ☐ | Crossbar routes transactions correctly between all master/slave pairs — cocotb integration test |
| --- | --- |
| ☐ | AXI-APB bridge handles back-pressure correctly — verify with slow-acknowledge APB peripheral |
| ☐ | CDC synchronisers in place for any cross-domain paths |
| ☐ | SymbiYosys proves all 4 SVA properties — show PASSED output |
| ☐ | Ordering test: issue 10 overlapping transactions, verify in-order response |
| ☐ | Commit to phase3_axi_interconnect/ folder |

Resume Bullet for Phase 3 Designed AXI4 3x3 crossbar interconnect with round-robin arbitration and AXI-APB bridge. Formally verified arbiter fairness, mutual exclusion, and starvation-freedom using SymbiYosys. Resolved CDC metastability with 2-FF synchronisers.

| Phase 4 Systolic Array AI Accelerator | ⏱ Weeks 5-8 |
| --- | --- |

This is the new project — the one that transforms your fundamentals portfolio into an AI hardware portfolio. A 4x4 systolic MAC array is attached to the SoC as a memory-mapped AXI4-Lite peripheral. The RISC-V core controls it via MMIO registers.

What is a Systolic Array? A systolic array is a grid of Processing Elements (PEs). Each PE does one multiply-accumulate (MAC). Data flows rhythmically between PEs like a heartbeat (systole). This is the architecture inside Google TPU, NVIDIA Tensor Cores, and every modern AI chip. Your 4x4 array can multiply a 4x4 matrix by a 4x4 matrix in 4+4+4-1 = 11 cycles — much faster than a scalar CPU.

### Key RTL Modules

| pe.sv | Single Processing Element: 8-bit IN × 8-bit weight → 32-bit accumulator, registered IO |
| --- | --- |
| systolic_array.sv | 4×4 grid of PEs, weight stationary dataflow, diagonal data feed |
| weight_buffer.sv | 16-entry register file holding 4×4 weight matrix (loaded once) |
| activation_buffer.sv | Input buffer, accepts 4 activations per cycle |
| output_buffer.sv | Accumulates 4×4 output matrix from PE array |
| axi_lite_slave.sv | AXI4-Lite slave: maps control/status/data registers to MMIO addresses |
| accel_top.sv | Top-level: connects all sub-blocks, exposes AXI4-Lite port to crossbar |

### MMIO Register Map

| Address | Register Name — Function |
| --- | --- |
| 0x5000_0000 | CTRL — bit[0]: start, bit[1]: reset, bit[2]: done (read-only) |
| 0x5000_0004 | STATUS — bit[0]: busy, bit[1]: result_valid |
| 0x5000_0008 to 0x5000_0044 | WEIGHT_REG[0..15] — 16 registers for 4×4 weight matrix (8-bit each) |
| 0x5000_0048 to 0x5000_0054 | INPUT_REG[0..3] — one column of activations per cycle |
| 0x5000_0058 to 0x5000_0094 | OUTPUT_REG[0..15] — 4×4 result matrix (32-bit accumulators) |

### Software Flow (RISC-V Firmware)

Write a simple C program (or assembly) that runs on your RISC-V core to use the accelerator:

- Step 1: Write 16 weight values to WEIGHT_REG[0..15] via store instructions

- Step 2: Write first column of activations to INPUT_REG[0..3]

- Step 3: Write 1 to CTRL[start]

- Step 4: Poll STATUS[busy] until 0

- Step 5: Read OUTPUT_REG[0..15] — compare against CPU-computed reference

### cocotb Verification Plan

- Test 1 — Identity matrix: multiply A × I = A — verifies data path correctness

- Test 2 — Known matrix: 4×4 multiply with hand-computed expected output

- Test 3 — Random stress: 100 random 4×4 multiplications compared against numpy reference

- Test 4 — AXI protocol: verify AWVALID/AWREADY handshake, BRESP always OKAY

- Test 5 — Back-to-back: start new computation immediately after done — no stale data

### Phase 4 Checklist

| ☐ | pe.sv implements MAC correctly — unit test in isolation first |
| --- | --- |
| ☐ | systolic_array.sv produces correct 4x4 matrix product — verify with identity matrix |
| ☐ | axi_lite_slave.sv handles all register reads and writes with correct AXI handshake |
| ☐ | All 5 cocotb tests pass with zero mismatches against numpy reference |
| ☐ | accel_top.sv integrates into the AXI crossbar as Slave 2 |
| ☐ | RISC-V firmware program runs and correctly offloads matrix multiply to accelerator |
| ☐ | Yosys synthesis of accel_top.sv — note area, compare against RISC-V core area |
| ☐ | Commit to phase4_ai_accelerator/ folder |

Resume Bullet for Phase 4 Designed 4x4 systolic array MAC accelerator (weight-stationary dataflow) in SystemVerilog. Integrated as AXI4-Lite MMIO peripheral to RISC-V SoC. Verified with cocotb against numpy reference across 100 random matrix multiplications. Synthesized on Sky130 PDK.

| Phase 5 Full SoC Integration + OpenLane Synthesis | ⏱ Weeks 8-10 |
| --- | --- |

Phase 5 is where everything comes together. You wire all blocks into a single top-level SoC, run a full integration test, and push the whole design through OpenLane to get real silicon metrics — area, timing, and power numbers for your resume.

### Top-Level SoC Architecture

Block Connectivity soc_top.sv ├── riscv_core (Phase 1) │ └── AXI4-Lite master ports (IMEM, DMEM) ├── cache_top (Phase 2) │ ├── CPU-side: AXI4-Lite slave │ └── Mem-side: AXI4 master → crossbar ├── axi4_crossbar (Phase 3) │ ├── Master 0: RISC-V IMEM │ ├── Master 1: Cache (DMEM) │ ├── Master 2: Accelerator DMA │ ├── Slave 0: SRAM (main memory) │ ├── Slave 1: APB peripherals │ └── Slave 2: AI Accelerator (Phase 4) └── accel_top (Phase 4) └── AXI4-Lite slave port ← from crossbar

### Integration Test — End-to-End

The flagship test: RISC-V core boots, loads a 4x4 matrix multiply program, offloads to the accelerator via MMIO, reads back the result, and checks it against a software reference. If this test passes, your SoC works.

- Step 1: Load test firmware into SRAM model via cocotb

- Step 2: Release RISC-V reset — core begins fetching instructions

- Step 3: Monitor AXI bus — verify RISC-V writes weights and activations to accelerator

- Step 4: Monitor STATUS register polling — verify RISC-V waits for done

- Step 5: Verify RISC-V reads correct output values from OUTPUT_REG

- Step 6: Check result against numpy reference in cocotb

### OpenLane Configuration

config.json (openlane/soc_top/) { "DESIGN_NAME": "soc_top", "VERILOG_FILES": "dir::../../phase5_integration/rtl/*.sv", "CLOCK_PORT": "clk", "CLOCK_PERIOD": 10.0, "FP_CORE_UTIL": 45, "PL_TARGET_DENSITY": 0.5 }

Run OpenLane cd openlane/ python -m openlane soc_top/config.json # Outputs: GDS, DEF, timing report, area report, power report # Find results in: runs/RUN_YYYY_MM_DD_*/

### Numbers to Extract for Resume

| Total Cell Area | From OpenLane area report — e.g. 0.34 mm² on Sky130 |
| --- | --- |
| Clock Frequency (Fmax) | From OpenSTA timing report — e.g. 75 MHz |
| Dynamic Power | From OpenROAD power report — e.g. 12 mW @ 1.8V |
| Integration Test | Pass/fail — '100% correct matrix multiply output' |
| Formal Proofs | 4 SVA properties proved for arbiter |
| Verification Coverage | cocotb tests — state count passed |

### Phase 5 Checklist

| ☐ | soc_top.sv compiles clean with verilator --lint-only |
| --- | --- |
| ☐ | End-to-end integration test passes: RISC-V correctly offloads and reads back matrix multiply result |
| ☐ | OpenLane run completes with zero DRC violations |
| ☐ | Extract and record: cell area, Fmax, dynamic power |
| ☐ | Save GDS screenshot to docs/ — this is your 'chip photo' |
| ☐ | Commit to phase5_integration/ and openlane/ folders |

Resume Bullet for Phase 5 (Full SoC) Integrated RISC-V core, L1 cache, AXI4 interconnect, and systolic array accelerator into a complete SoC. Full end-to-end simulation via cocotb (RISC-V firmware offloads matrix multiply to accelerator, result verified correct). Synthesized with OpenLane on Sky130 130nm PDK: [X] mm² area, [X] MHz Fmax, [X] mW dynamic power.

| Phase 6 Documentation, GitHub & Interview Prep | ⏱ Weeks 10-11 |
| --- | --- |

The SoC is built. Now you package it so that a recruiter who spends 90 seconds on your GitHub immediately understands what you did and why it matters.

### README.md Structure

- Header: Project title + one-line summary + tech stack badges

- Section 1: The Story — 'I built one SoC incrementally across 5 phases'

- Section 2: Architecture diagram (draw in draw.io, export as PNG)

- Section 3: GDS screenshot from OpenLane

- Section 4: Results table — area, Fmax, power, test coverage

- Section 5: Phase breakdown — one paragraph per phase with what you built and why

- Section 6: How to run — single command to simulate, single command to synthesize

### How to Present This in Interviews

The 30-Second Pitch "I built a complete RISC-V AI SoC from scratch using open-source tools. It started as a bare 5-stage pipeline, then I added an L1 cache, connected everything through an AXI4 crossbar, and finally attached a systolic array accelerator — the same architecture used in Google TPU. The RISC-V core can offload matrix multiply to the accelerator via memory-mapped registers. I formally verified the arbiter, ran end-to-end cocotb tests, and synthesized the full system through OpenLane: [X] mm², [X] MHz, [X] mW."

### Interview Questions to Prepare For

| What is a systolic array and why is it efficient for matrix multiply? | Data reuse — each activation passes through a whole row of PEs, each weight stays stationary. No memory bandwidth bottleneck. |
| --- | --- |
| How does AXI4 handle back-pressure? | VALID/READY handshake — master asserts VALID, slave asserts READY, transfer occurs only when both high simultaneously. |
| What is the difference between formal and simulation verification? | Formal exhaustively proves properties for ALL inputs; simulation only checks the inputs you write test cases for. |
| What is a write-back cache and why is it better than write-through? | Write-back holds dirty data in cache and only writes to memory on eviction — fewer memory writes, better bandwidth. |
| Why did you use cocotb instead of UVM? | cocotb is Python-native, easier to integrate with numpy for golden models, and increasingly adopted in industry especially for AI hardware. |
| What are your OpenLane synthesis numbers? | State your actual numbers — area in mm², Fmax in MHz, power in mW. |

# Master Timeline

| Week 1 — Days 1-2 | Phase 0: Repo setup, tool installation, folder structure |
| --- | --- |
| Week 1-3 | Phase 1: RISC-V core cleanup, cocotb migration, Yosys synthesis |
| Week 3-4 | Phase 2: Cache integration, hit/miss analysis, cocotb golden model |
| Week 4-5 | Phase 3: AXI crossbar integration, SymbiYosys formal proofs |
| Week 5-8 | Phase 4: Systolic array design, AXI wrapper, numpy verification |
| Week 8-10 | Phase 5: SoC integration, end-to-end test, OpenLane tape-out |
| Week 10-11 | Phase 6: README, architecture diagram, interview prep |
| Week 11+ | Apply — with a complete, synthesized, formally verified AI SoC |

Final Resume Block (One Project, Maximum Impact) RISC-V AI SoC — Full-Stack RTL Design & Verification SystemVerilog · cocotb · SymbiYosys · Yosys · OpenLane · Sky130 PDK Designed and verified a complete SoC from scratch across 5 incremental phases: Phase 1 — 5-stage RV32I pipeline (hazard detection, forwarding, branch resolution) Phase 2 — Direct-mapped write-back L1 cache; 95% hit rate under sequential workload Phase 3 — AXI4 3x3 crossbar + AXI-APB bridge; arbiter formally verified (SymbiYosys) Phase 4 — 4x4 systolic MAC array as AXI4-Lite MMIO accelerator; verified vs numpy Phase 5 — Full SoC integration; RISC-V firmware offloads matrix multiply to accelerator Synthesized with OpenLane on Sky130 130nm: [X] mm² · [X] MHz · [X] mW dynamic power

Build it. Commit it. Own it.