RISC-V AI SoC

From-Scratch Project Blueprint

Complete Phase-by-Phase Execution Roadmap for VLSI + AI Fresher Placement

What You Are Building One complete SoC — not five separate projects. RISC-V Core → L1 Cache → AXI4 Interconnect → AI Accelerator → Full Integration Every module is one layer of the same chip. You build it incrementally, phase by phase, from nothing to a fully synthesized AI SoC with real silicon numbers from OpenLane. Timeline: ~18 weeks ¦ Tools: 100% open-source ¦ Result: One flagship portfolio project

# Open-Source Tool Stack

All tools below are free, run on Ubuntu, and produce industry-equivalent outputs. Recruiters care about methodology and results — not whether you used Synopsys or Yosys.

| VS Code + Extensions | RTL writing — install: Verilog-HDL, SystemVerilog, and WaveTrace extensions |
| --- | --- |
| Verilator | Linting: verilator --lint-only catches type errors before simulation |
| Icarus Verilog | Simulation: iverilog compiles, vvp runs, GTKWave shows waveforms |
| cocotb | Python-based verification framework — replaces UVM, modern and industry-adopted |
| SymbiYosys | Formal verification — replaces JasperGold, uses SVA properties |
| Yosys | RTL synthesis — produces gate-level netlist from SystemVerilog |
| OpenSTA | Static timing analysis — replaces PrimeTime, gives Fmax |
| OpenROAD | Place and route — gives real layout from netlist |
| OpenLane | Full RTL-to-GDSII flow wrapping Yosys + OpenROAD — run once for all reports |
| Sky130 PDK | Google's open 130nm process — real foundry rules, used in actual tapeouts |
| GTKWave | Waveform viewer for simulation dumps |
| Python + numpy | Golden model computation for verification comparison |

Install Everything (Ubuntu — one time setup) sudo apt update sudo apt install iverilog gtkwave verilator yosys sudo apt install symbiyosys yosys-smtbmc pip install cocotb openlane # OpenLane downloads Sky130 PDK automatically on first run (~2 GB)

# Master Timeline Overview

| Phase 0 | Repo setup + tool install + learning SystemVerilog basics |
| --- | --- |
| Phase 1 | RISC-V RV32I 5-stage pipeline — the CPU core |
| Phase 2 | L1 Cache Controller — direct-mapped write-back |
| Phase 3 | AXI4 Interconnect + AXI-APB Bridge — the SoC backbone |
| Phase 4 | AI Accelerator — 4×4 systolic MAC array |
| Phase 5 | Full SoC Integration — wire everything together |
| Phase 6 | OpenLane Synthesis — get real silicon numbers |
| Phase 7 | Documentation + GitHub + Interview Prep |

| Total Duration | ~18 weeks (4.5 months) — building everything from scratch |
| --- | --- |
| Hours per week | 10–15 hrs/week — sustainable alongside job applications |
| Difficulty ramp | Phases 0-2 are learnable; Phases 3-4 are challenging; Phase 5-6 are integration |
| Minimum viable | Phases 1-3 alone are already a strong portfolio — Phases 4-6 make it exceptional |

⚠️ Common Mistake Do NOT try to build all phases simultaneously. Complete each phase fully, commit it, and verify it works before moving to the next. The whole system fails if any layer is broken.

| PHASE 0 Environment Setup + SystemVerilog Basics | ⏱ Weeks 1-2 |
| --- | --- |

Before writing a single hardware module, you need a working development environment and enough SystemVerilog knowledge to write and simulate basic RTL. Do not skip this phase — getting stuck on tooling mid-project wastes far more time.

## Step 1 — Install All Tools

Run the install commands from the tool stack section above. Verify each tool:

Verification Commands iverilog -V # should print Icarus Verilog version verilator --version # should print Verilator version yosys --version # should print Yosys version python -c 'import cocotb; print(cocotb.__version__)' sby --help # SymbiYosys help

## Step 2 — Set Up GitHub Repository

Create a public GitHub repo called riscv-ai-soc. This is what recruiters will browse. Set up this folder structure from day one:

Repository Folder Structure riscv-ai-soc/ ├── README.md ├── docs/ ← architecture diagrams, waveform screenshots ├── phase1_riscv_core/ │ ├── rtl/ ← all .sv RTL source files │ ├── tb/ ← cocotb testbench Python files │ ├── formal/ ← SymbiYosys scripts and SVA files │ └── synth/ ← Yosys synthesis scripts and reports ├── phase2_cache/ │ ├── rtl/ tb/ synth/ ├── phase3_axi/ │ ├── rtl/ tb/ formal/ synth/ ├── phase4_accelerator/ │ ├── rtl/ tb/ synth/ ├── phase5_integration/ │ ├── rtl/ tb/ └── openlane/ └── soc_top/ ← OpenLane config and reports

## Step 3 — Learn SystemVerilog Basics (If Needed)

If you are new to SystemVerilog, spend the first week on these specific topics only. Do not try to learn everything — learn exactly what you need for Phase 1.

| Modules and ports | module, input, output, logic, wire, reg |
| --- | --- |
| Combinational logic | always_comb, assign, if-else, case |
| Sequential logic | always_ff @(posedge clk), non-blocking <= |
| Parameters | parameter, localparam — needed for parameterized designs |
| Structs and enums | typedef struct, typedef enum — used in pipeline registers |
| Simulation basics | $display, $monitor, $finish, $dumpvars for GTKWave |
| 💡 Tip Best free resource: hdlbits.01xz.net — interactive browser-based SV exercises. Complete the 'Verilog Language' and 'Circuits > Sequential Logic' sections before Phase 1. Target: finish HDLBits basics in week 1, start Phase 1 RTL in week 2. |

## Phase 0 Completion Checklist

| ☐ | All tools installed and version-verified |
| --- | --- |
| ☐ | GitHub repo created: riscv-ai-soc (public) |
| ☐ | Folder structure created and pushed |
| ☐ | Can write a simple D flip-flop in SV and simulate it with iverilog |
| ☐ | Can view a waveform in GTKWave |
| ☐ | Completed at least: HDLBits — Getting Started, Verilog Language sections |

| PHASE 1 5-Stage RISC-V RV32I Pipeline Core | ⏱ Weeks 2-7 |
| --- | --- |

This is the heart of the SoC. A 5-stage in-order pipeline implementing the RV32I base integer ISA. Everything else in the project connects to this CPU. Build it stage by stage — do not try to wire all 5 stages at once.

What is RISC-V RV32I? RISC-V is an open-source ISA (instruction set architecture). RV32I is the base integer subset: 32-bit registers, 32-bit addresses, 47 instructions. It is simple enough to implement from scratch but complete enough to run real programs. Your 5 stages are: IF (Instruction Fetch) → ID (Decode) → EX (Execute) → MEM (Memory) → WB (Writeback)

## Build Order — Stage by Stage

Build in this exact order. Simulate and verify each stage before adding the next.

### Week 2-3: ALU + Register File (Foundation)

Start here, not with the pipeline. These two modules underpin everything.

| alu.sv | Inputs: a, b (32-bit), alu_ctrl (4-bit). Outputs: result (32-bit), zero flag. Operations: ADD, SUB, AND, OR, XOR, SLL, SRL, SRA, SLT, SLTU. Use always_comb + case. |
| --- | --- |
| reg_file.sv | 32 registers × 32-bit. Two async read ports (rs1, rs2). One sync write port (rd). Register x0 always reads 0 — hardwire this. Simulate: write r1=5, read back r1. |
| 💡 Tip Test ALU first in isolation: write a cocotb test that sends 20 operation pairs and checks results against Python arithmetic. If your ALU is wrong, every later stage that uses it will also be wrong. |

### Week 3-4: IF and ID Stages

| fetch_stage.sv | PC register (starts at 0). Instruction memory read (synchronous). Branch mux: PC+4 (normal) or branch_target (taken). Output: instruction word + PC value to ID stage. |
| --- | --- |
| decode_stage.sv | Decodes instruction fields: opcode, rs1, rs2, rd, funct3, funct7. Reads register file. Generates all control signals: reg_write, mem_read, mem_write, alu_src, branch, jump. Generates immediate (sign-extended): I-type, S-type, B-type, U-type, J-type. |
| imm_gen.sv | Separate module just for immediate generation. Case on opcode. 6 immediate types in RV32I. Get this right — it is a common bug source. |
| pipeline_reg_IF_ID.sv | Register that holds IF stage outputs, clocked. Needed for stall logic later. |

### Week 4-5: EX, MEM, WB Stages

| execute_stage.sv | ALU source mux (register or immediate). Branch condition evaluation (BEQ, BNE, BLT, BGE, BLTU, BGEU). Branch target address = PC + imm. JAL/JALR handling. Forwarding mux (added in Week 5). |
| --- | --- |
| memory_stage.sv | Data memory read (lw, lh, lb) and write (sw, sh, sb). Sign extension on loads. Expose AXI4-Lite master interface — this will connect to your cache in Phase 2. |
| writeback_stage.sv | Mux: write ALU result, memory load result, or PC+4 (for JAL) back to register file. |
| pipeline_reg_*.sv | One pipeline register module per stage boundary: IF_ID, ID_EX, EX_MEM, MEM_WB. |

### Week 5-6: Hazard Detection + Forwarding

This is the hardest part of Phase 1. Do not skip it — hazard handling is always asked in interviews.

| hazard_unit.sv | Detects: load-use hazards (1 stall cycle needed). Generates: stall signal (freeze IF/ID registers) and flush signal (insert NOP bubble into ID/EX). Also detects control hazards on branches. |
| --- | --- |
| forwarding_unit.sv | Detects RAW (read-after-write) data hazards. Generates forwarding mux selects: forward from EX/MEM or MEM/WB stage. Covers: EX-EX forwarding and MEM-EX forwarding paths. |
| ⚠️ Common Mistake Do NOT try to implement out-of-order or speculative execution. Keep it simple: static not-taken branch prediction, one stall cycle on load-use. That is enough for a fresher portfolio. |

### Week 6-7: Top-Level + cocotb Verification

| riscv_core.sv | Top-level that instantiates and wires all stage modules. Exposes: clk, rst, instruction memory port, data memory port (AXI4-Lite). This is the module that plugs into the SoC. |
| --- | --- |
| test_alu.py | cocotb: 20 arithmetic ops vs Python reference |
| test_instructions.py | cocotb: hand-assembled RV32I program — tests add, sub, lw, sw, beq, jal |
| test_hazards.py | cocotb: sequence with RAW hazard — verify forwarding corrects result without stall |
| test_load_use.py | cocotb: lw followed immediately by dependent instruction — verify 1 stall inserted |
| test_bubblesort.py | cocotb: 8-element bubble sort in assembly — end-to-end correctness check |

## Yosys Synthesis (After Verification Passes)

Synthesis Script — phase1_riscv_core/synth/synth.ys read_verilog -sv ../rtl/riscv_core.sv synth -top riscv_core dfflibmap -liberty /path/to/sky130_fd_sc_hd__tt_025C_1v80.lib abc -liberty /path/to/sky130_fd_sc_hd__tt_025C_1v80.lib stat # Prints: number of cells, estimated area write_verilog synth/riscv_core_netlist.v

## Phase 1 Completion Checklist

| ☐ | alu.sv: all 10 operations correct — cocotb test passes |
| --- | --- |
| ☐ | reg_file.sv: read/write correct, x0 always 0 |
| ☐ | imm_gen.sv: all 5 immediate types correct |
| ☐ | All 5 stages implemented and wired in riscv_core.sv |
| ☐ | Pipeline registers (IF_ID, ID_EX, EX_MEM, MEM_WB) implemented |
| ☐ | hazard_unit.sv: load-use stall works — verified with test_load_use.py |
| ☐ | forwarding_unit.sv: EX-EX and MEM-EX forwarding works — verified with test_hazards.py |
| ☐ | test_bubblesort.py passes — array correctly sorted end-to-end |
| ☐ | Yosys synthesis completes — note cell count and critical path |
| ☐ | GTKWave waveform captured — screenshot saved to docs/ |
| ☐ | Committed to phase1_riscv_core/ with descriptive commit messages |

✅ Resume Bullet — Copy This After Completing the Phase Designed 5-stage pipelined RISC-V RV32I processor in SystemVerilog from scratch. Implements full hazard detection unit, data forwarding (EX-EX, MEM-EX paths), and static branch prediction. Verified with cocotb: instruction-level tests + bubble sort end-to-end. Synthesized with Yosys on Sky130.

🎯 Interview Questions for This Phase Q: What is a data hazard and how does forwarding solve it? Q: What is a load-use hazard and why can it not be solved by forwarding alone? Q: What is the difference between a structural, data, and control hazard? Q: Walk me through what happens in your pipeline on a branch instruction. Q: What is the CPI of your processor under a workload with 20% branches, 10% loads?

| PHASE 2 Direct-Mapped Write-Back L1 Cache Controller | ⏱ Weeks 7-9 |
| --- | --- |

The cache sits between your RISC-V core's data memory port and main memory. Without it, every lw/sw would take multiple cycles waiting for SRAM. The cache makes most accesses 1 cycle by keeping frequently used data close to the CPU.

Cache Basics You Must Know Before Starting Direct-mapped: each main memory address maps to exactly one cache set. Write-back: dirty data is NOT written to memory on every write — only on eviction. Dirty bit: marks a cache line that has been written but not yet evicted to memory. Valid bit: marks a cache line that contains valid data (0 on reset). Tag: the upper bits of the address that identify which memory block is in this set. Hit: address tag matches and valid=1. Miss: tag mismatch or valid=0.

## Cache Parameters (Design Choices)

| Cache size | 4 KB total |
| --- | --- |
| Block size | 16 bytes (4 words of 32-bit each) |
| Number of sets | 64 sets (4096 / (64 / 4) = 64... direct-mapped so 1 way) |
| Address breakdown | 32-bit address: [31:12] tag ¦ [11:6] index (6 bits=64) ¦ [5:2] word offset ¦ [1:0] byte offset |
| Write policy | Write-back + write-allocate |
| Replacement | Direct-mapped — no replacement policy needed (only one way) |

## RTL Modules to Build

| cache_tag_array.sv | 64-entry array of {tag[19:0], valid, dirty}. Synchronous write, async read. Implement as a 2D logic array. |
| --- | --- |
| cache_data_array.sv | 64×128-bit array (64 sets, 4 words per line). Synchronous write per word, async read. Implement as a 2D logic array. |
| cache_controller.sv | Main FSM. States: IDLE → TAG_CHECK → (HIT path) → WRITE_BACK → FILL → RESPOND. Handles: hit return, clean miss fill, dirty eviction+fill. |
| cache_top.sv | Top-level: CPU-side AXI4-Lite slave port + memory-side AXI4 master port. Connects controller, tag array, data array. This plugs into the AXI crossbar in Phase 3. |

## Cache FSM — State by State

| IDLE | Wait for CPU request (AXI ARVALID or AWVALID) |
| --- | --- |
| TAG_CHECK | Read tag array. Compare tag bits. Check valid bit. Next state: HIT or MISS |
| HIT | Read/write data array. Return data to CPU in 1 cycle. Back to IDLE. |
| WRITE_BACK | If dirty on miss: write dirty cache line to memory via AXI4 burst. Go to FILL. |
| FILL | Fetch new cache line from memory via AXI4 burst (4 words). Write to data+tag array. Go to RESPOND. |
| RESPOND | Return filled data to CPU. Back to IDLE. |

## cocotb Verification Plan

| test_read_hit.py | Fill cache by writing 10 addresses. Read them back. All must return in 1 cycle (hit). |
| --- | --- |
| test_cold_miss.py | Fresh reset. Read 10 new addresses. All must miss first time, hit on second read. |
| test_dirty_eviction.py | Write to address A (makes it dirty). Access conflicting address (same set, different tag). Verify: write-back to memory occurs before fill. Check memory has the old dirty data. |
| test_golden_model.py | 1000 random read/write addresses. Compare every response against a Python dict-based cache model. Zero mismatches required. |
| test_hit_rate.py | Four access pattern workloads: sequential, strided-8, random, looping. Print hit rate for each. Save table to docs/cache_analysis.txt. |

## Python Golden Model (for test_golden_model.py)

Golden Model — cache_model.py class CacheModel: def __init__(self): self.sets = [{} for _ in range(64)] # dict: tag -> (data, dirty) self.memory = {} # word_address -> value def read(self, addr): idx = (addr >> 4) & 0x3F # 6-bit index tag = addr >> 10 # 20-bit tag if tag in self.sets[idx]: # hit return self.sets[idx][tag][0] else: # miss — fill from memory data = self.memory.get(addr, 0) self.sets[idx][tag] = (data, False) return data def write(self, addr, data): idx = (addr >> 4) & 0x3F tag = addr >> 10 self.sets[idx][tag] = (data, True) # mark dirty

## Phase 2 Completion Checklist

| ☐ | cache_controller.sv FSM: all 6 states implemented |
| --- | --- |
| ☐ | cache_tag_array.sv and cache_data_array.sv: read/write working in isolation |
| ☐ | test_read_hit.py: 100% hits after warmup — 1-cycle latency confirmed |
| ☐ | test_cold_miss.py: first access always misses, second always hits |
| ☐ | test_dirty_eviction.py: write-back to memory confirmed via AXI bus monitor |
| ☐ | test_golden_model.py: 1000 random ops — zero golden model mismatches |
| ☐ | Hit rate table generated and saved to docs/cache_analysis.txt |
| ☐ | cache_top.sv integrates with RISC-V data memory port — end-to-end lw/sw test passes |
| ☐ | Yosys synthesis of cache_top.sv — note area |
| ☐ | Committed to phase2_cache/ folder |

✅ Resume Bullet — Copy This After Completing the Phase Implemented direct-mapped write-back L1 cache (4KB, 64 sets, 16B blocks) in SystemVerilog. Cache controller FSM handles hit, clean miss, and dirty eviction+fill paths. Verified with cocotb against Python golden model (1000 random ops, zero mismatches). Hit rate: 95% sequential, 30% random. Integrated with RISC-V core via AXI4-Lite interface.

🎯 Interview Questions for This Phase Q: What is the difference between write-back and write-through? Which is better for performance? Q: What happens on a dirty cache miss? Walk me through all the steps. Q: What is write-allocate vs write-no-allocate? Q: Why does strided access have a lower hit rate than sequential? Q: What is cache thrashing and how does it happen in a direct-mapped cache?

| PHASE 3 AXI4 Crossbar Interconnect + AXI-APB Bridge | ⏱ Weeks 9-12 |
| --- | --- |

The interconnect is the SoC backbone. All communication between your CPU, cache, accelerator, and memory goes through this crossbar. You also add a formal verification layer for the arbiter — this is a strong differentiator for a fresher resume.

AXI4 Protocol — What You Must Know First AXI4 has 5 channels: AW (write address), W (write data), B (write response), AR (read address), R (read data). Each channel uses a VALID/READY handshake: transfer happens only when BOTH are high in the same cycle. Master drives VALID (I have data). Slave drives READY (I can accept data). AXI4-Lite: simplified version — no bursts, single transaction at a time. Used for register-mapped peripherals. AXI4: full version — supports burst transfers up to 256 beats. Used for memory and DMA.

## SoC Connectivity Map

| Master 0 | RISC-V instruction fetch port (AXI4-Lite) |
| --- | --- |
| Master 1 | RISC-V data memory port through L1 cache (AXI4) |
| Master 2 | AI Accelerator DMA port — added in Phase 4 (AXI4-Lite) |
| Slave 0 | Main SRAM memory — 64KB behavioural model (AXI4) |
| Slave 1 | APB peripheral space via AXI-APB bridge (AXI4-Lite → APB) |
| Slave 2 | AI Accelerator register map — MMIO control (AXI4-Lite, added Phase 4) |

## RTL Modules to Build

| axi4_arbiter.sv | Round-robin arbiter: grants access to one master per cycle. Inputs: request vector. Outputs: grant vector. THIS module will be formally verified with SymbiYosys. |
| --- | --- |
| axi4_crossbar.sv | 3-master × 3-slave routing matrix. Uses axi4_arbiter for each slave port. Address decoding: top bits of AWADDR/ARADDR select which slave. |
| axi4_apb_bridge.sv | Protocol converter: AXI4-Lite write → APB SETUP+ENABLE phases. AXI4-Lite read → APB read. Handles PREADY back-pressure: holds AXI response until PREADY high. |
| cdc_sync.sv | 2-FF synchroniser for any signal crossing clock domains. Use for any async control signals between blocks. |
| addr_decoder.sv | Combinational: takes AXI AWADDR/ARADDR, outputs slave_select. Define memory map here. Simple case statement. |
| sram_model.sv | Behavioural 64KB AXI4 slave. Array of logic [31:0] mem [0:16383]. Used for simulation only — replaced by real SRAM in synthesis. |

## Memory Map (Address Decoder)

| 0x0000_0000 – 0x0000_FFFF | SRAM — main memory (64 KB) → Slave 0 |
| --- | --- |
| 0x1000_0000 – 0x1000_FFFF | APB peripherals → Slave 1 (via bridge) |
| 0x5000_0000 – 0x5000_00FF | AI Accelerator registers → Slave 2 (Phase 4) |

## Formal Verification of Arbiter — SymbiYosys

Write these four SVA (SystemVerilog Assertion) properties inside axi4_arbiter.sv and prove them with SymbiYosys:

| Mutual Exclusion | assert property (@(posedge clk) $onehot0(grant)): at most one master granted at a time |
| --- | --- |
| Grant Stability | assert property: once granted, grant held until transaction completes (AXI VALID+READY seen) |
| No Phantom Grants | assert property: grant only asserted when corresponding request is high |
| Fairness (Liveness) | cover property: every requesting master receives a grant within 8 cycles |
| SymbiYosys Script — phase3_axi/formal/arbiter.sby [options] mode prove depth 20 [engines] smtbmc boolector [script] read -formal ../rtl/axi4_arbiter.sv prep -top axi4_arbiter [files] ../rtl/axi4_arbiter.sv # Run with: sby arbiter.sby # PASSED = all assertions proved for all inputs up to depth 20 |

## cocotb Verification Plan

| test_single_master.py | One master sends 10 read and 10 write transactions to SRAM. Verify correct responses. |
| --- | --- |
| test_concurrent.py | All 3 masters send requests simultaneously. Verify each gets a grant, no data corruption. |
| test_back_pressure.py | SRAM model asserts RREADY=0 for 5 cycles. Verify master waits without data loss. |
| test_bridge_write.py | AXI write to APB address. Monitor APB bus: verify PSEL, PENABLE, PWRITE sequence. |
| test_bridge_read.py | AXI read from APB address with slow PREADY (3-cycle delay). Verify correct data returned. |
| test_ordering.py | Issue 20 transactions from same master. Verify responses return in order. |

## Phase 3 Completion Checklist

| ☐ | axi4_arbiter.sv: round-robin logic correct — all 3 masters get grants in rotation |
| --- | --- |
| ☐ | SymbiYosys: all 4 SVA properties PROVED — screenshot/log saved to docs/ |
| ☐ | axi4_crossbar.sv: routes correctly between all master/slave combinations |
| ☐ | axi4_apb_bridge.sv: SETUP+ENABLE sequence correct, back-pressure handled |
| ☐ | cdc_sync.sv: 2-FF synchroniser implemented for any cross-domain paths |
| ☐ | test_concurrent.py: no data corruption under simultaneous masters |
| ☐ | test_ordering.py: 20 transactions return in order |
| ☐ | Memory map implemented in addr_decoder.sv — verified with correct slave routing |
| ☐ | Committed to phase3_axi/ folder |

✅ Resume Bullet — Copy This After Completing the Phase Designed AXI4 3×3 crossbar interconnect with round-robin arbitration and AXI4-to-APB bridge. Formally verified arbiter fairness, mutual exclusion, and grant stability using SymbiYosys (4 SVA properties proved). Handled back-pressure and CDC metastability with 2-FF synchronisers.

🎯 Interview Questions for This Phase Q: Explain the AXI4 VALID/READY handshake. What happens if VALID is high but READY is low? Q: What is the difference between AXI4 and AXI4-Lite? Q: What is the difference between simulation-based and formal verification? Q: What is CDC and why is it a problem in SoC design? Q: How does a round-robin arbiter ensure fairness?

| PHASE 4 AI Accelerator — 4×4 Systolic MAC Array | ⏱ Weeks 12-15 |
| --- | --- |

This is the project that makes your portfolio stand out. A systolic array is the core architecture of Google TPU, NVIDIA Tensor Cores, and every major AI chip. You are building a small but architecturally correct version and integrating it into your SoC.

How a Systolic Array Works A systolic array is a grid of identical Processing Elements (PEs). Each PE does ONE multiply-accumulate. Data flows rhythmically between PEs like a heartbeat (from the Greek 'systole'). Weight-stationary dataflow: weights are loaded once into PEs and stay fixed. Activations are fed in one column per cycle and flow across the PE rows. Each PE computes: accumulator += activation_in × weight, passes activation_in to next PE. A 4×4 array computes a full 4×4 matrix multiply in 4+4-1 = 7 cycles (diagonal feeding). This is orders of magnitude more efficient than a scalar CPU doing 64 multiply-accumulates sequentially.

## RTL Modules to Build

| pe.sv | Single Processing Element. Inputs: act_in (8-bit), weight (8-bit, loaded at config time). Output: act_out (passes act_in to next PE), accum (32-bit running sum). Registered IO. Core logic: accum <= accum + (act_in * weight) |
| --- | --- |
| systolic_array.sv | 4×4 instantiation of pe.sv. Wires act_out of PE[i][j] to act_in of PE[i][j+1]. Wires separate partial sum accumulation column by column. Diagonal activation feeding: act input for row i is delayed by i cycles. |
| delay_chain.sv | Parameterized shift-register chain to implement diagonal feeding. PE row 0 gets act cycle 0, row 1 gets act cycle 1 (one D-FF delay), etc. |
| weight_buffer.sv | 16-register array (4×4), one 8-bit register per PE weight. Loaded via configuration interface before computation starts. |
| output_buffer.sv | Captures 4×4 accumulator outputs when done signal asserted. Holds result until CPU reads via AXI. |
| axi_lite_slave.sv | AXI4-Lite slave wrapping the accelerator. Maps weight registers, input registers, output registers, and control/status to MMIO addresses. Generates start signal, captures done signal. |
| accel_top.sv | Top-level: connects weight_buffer, delay_chain, systolic_array, output_buffer, and axi_lite_slave. Exposes single AXI4-Lite port to the crossbar. |

## MMIO Register Map

| 0x5000_0000 CTRL | Bit 0: start (write 1 to begin), Bit 1: reset |
| --- | --- |
| 0x5000_0004 STATUS | Bit 0: busy, Bit 1: result_valid (read-only) |
| 0x5000_0008 WEIGHT_BASE | 16 registers × 4 bytes = 64 bytes for 4×4 weight matrix |
| 0x5000_0048 INPUT_BASE | 4 registers × 4 bytes = 16 bytes for one activation column |
| 0x5000_0058 OUTPUT_BASE | 16 registers × 4 bytes = 64 bytes for 4×4 result matrix (32-bit accumulators) |

## Build Order for Phase 4

- Build and test pe.sv in complete isolation first. Write a cocotb unit test: load weight=3, feed 4 activations (1,2,3,4), verify accum = 3+6+9+12 = 30.

- Build systolic_array.sv with all 16 PEs. Test with identity weight matrix: output should equal input.

- Add delay_chain.sv for diagonal activation feeding. Test: feed activations [1,1,1,1] each cycle, verify all 4 rows receive their delayed copies.

- Build weight_buffer.sv and output_buffer.sv. Test register read/write in isolation.

- Build axi_lite_slave.sv. Test each register address: write to WEIGHT_BASE[0], read it back, verify correct value.

- Wire accel_top.sv. Run full matrix multiply test against numpy.

- Connect accel_top to AXI crossbar as Slave 2 from Phase 3.

## cocotb Verification Plan

| test_pe_unit.py | Weight=3, feed activations 1-4, verify accumulator = 30. Weight=0, verify output always 0. |
| --- | --- |
| test_identity.py | Load 4×4 identity matrix as weights. Feed input matrix A. Verify output == A (identity property). |
| test_known_matrix.py | Hand-compute 4×4 × 4×4 multiplication. Load into accelerator. Verify element-by-element. |
| test_numpy_random.py | 100 iterations: random 8-bit 4×4 matrices A,B. Compute A×B in numpy. Load A into weights, B columns into inputs. Verify accelerator output matches numpy. Zero mismatches required. |
| test_axi_protocol.py | Verify AXI handshake on every register access: AWVALID/AWREADY, WVALID/WREADY, BVALID. Check BRESP=2'b00 (OKAY) on all valid writes. |
| test_back2back.py | Start computation, wait for done, immediately start a new computation. Verify no stale data from previous run. |

## Phase 4 Completion Checklist

| ☐ | pe.sv: unit test passes — accumulator correct for 5 different weight/activation combinations |
| --- | --- |
| ☐ | systolic_array.sv: identity matrix test passes |
| ☐ | delay_chain.sv: diagonal feeding verified — each row receives activation with correct delay |
| ☐ | test_known_matrix.py: hand-verified matrix multiply correct |
| ☐ | test_numpy_random.py: 100 random 4×4 multiplications — zero mismatches |
| ☐ | test_axi_protocol.py: all AXI handshakes correct, BRESP always OKAY |
| ☐ | test_back2back.py: no stale data between back-to-back computations |
| ☐ | accel_top.sv connected to AXI crossbar as Slave 2 — crossbar routes to it correctly |
| ☐ | Yosys synthesis of accel_top.sv — note area (compare with RISC-V core for context) |
| ☐ | Committed to phase4_accelerator/ folder |

✅ Resume Bullet — Copy This After Completing the Phase Designed 4×4 systolic array MAC accelerator (weight-stationary dataflow) in SystemVerilog. Integrated as AXI4-Lite MMIO peripheral in RISC-V SoC. Verified with cocotb against numpy reference: 100 random 4×4 INT8 matrix multiplications, zero mismatches. Synthesized on Sky130.

🎯 Interview Questions for This Phase Q: What is a systolic array and why is it efficient for matrix multiplication? Q: What is weight-stationary dataflow? What are the alternatives? Q: Why use INT8 weights instead of FP32 in inference accelerators? Q: How does your accelerator communicate with the CPU? Walk me through a matrix multiply. Q: What is the theoretical throughput of your 4×4 array in MACs/cycle?

| PHASE 5 Full SoC Integration | ⏱ Weeks 15-16 |
| --- | --- |

Every block is built and individually verified. Phase 5 wires them all into one soc_top.sv and runs the end-to-end integration test: RISC-V firmware executes a matrix multiply by talking to the accelerator over the AXI bus. If this passes, your SoC works.

## soc_top.sv — Instantiation List

| riscv_core | Phase 1 — AXI4-Lite master ports for IMEM and DMEM |
| --- | --- |
| cache_top | Phase 2 — CPU-side AXI4-Lite slave, mem-side AXI4 master |
| axi4_crossbar | Phase 3 — 3 masters, 3 slaves, round-robin arbitration |
| sram_model | Phase 3 — 64KB behavioural memory, AXI4 slave |
| axi4_apb_bridge | Phase 3 — AXI4-Lite to APB converter |
| accel_top | Phase 4 — AI accelerator, AXI4-Lite slave |

## End-to-End Integration Test

Write a simple RISC-V assembly program that does a matrix multiply using the accelerator:

firmware.s — RISC-V Assembly Firmware # Step 1: Write 16 weights to WEIGHT_BASE (0x50000008) li t0, 0x50000008 li t1, 3 # weight value sw t1, 0(t0) # WEIGHT[0] = 3 # ... repeat for all 16 weights # Step 2: Write input activations to INPUT_BASE (0x50000048) li t0, 0x50000048 li t1, 1 sw t1, 0(t0) # INPUT[0] = 1 # ... repeat for all 4 inputs # Step 3: Start the accelerator li t0, 0x50000000 li t1, 1 sw t1, 0(t0) # CTRL = 1 (start) # Step 4: Poll STATUS until done poll: lw t1, 4(t0) # read STATUS andi t1, t1, 2 # check result_valid bit beqz t1, poll # Step 5: Read results from OUTPUT_BASE (0x50000058) li t0, 0x50000058 lw a0, 0(t0) # OUTPUT[0] # ... read all 16 outputs

## Integration Test with cocotb

| test_integration.py | Load firmware.s binary into SRAM model at address 0. Release RISC-V reset. Monitor AXI bus. After firmware completes, read accelerator output registers. Compare against numpy. This is your flagship test. |
| --- | --- |
| test_memory_map.py | Verify address decoder: write to each slave's address range, confirm transaction reaches correct slave and no other. |
| test_cache_hit_rate.py | Run firmware multiple times. Count cache hits on IMEM fetches — instruction loop should be nearly 100% hits after first iteration. |

## Phase 5 Completion Checklist

| ☐ | soc_top.sv: all 6 blocks instantiated and wired correctly — compiles with verilator --lint-only |
| --- | --- |
| ☐ | test_integration.py: RISC-V firmware runs to completion, accelerator returns correct matrix multiply result |
| ☐ | test_memory_map.py: address decoder routes every address range to correct slave |
| ☐ | End-to-end waveform captured in GTKWave — shows RISC-V fetch, AXI transactions, accelerator operation |
| ☐ | Committed to phase5_integration/ folder |

| PHASE 6 OpenLane Synthesis — Real Silicon Numbers | ⏱ Weeks 16-17 |
| --- | --- |

This phase turns your RTL into a real physical layout and produces the area, timing, and power numbers that go on your resume. These numbers make your project verifiable and credible — a recruiter can clone your repo and reproduce them.

## OpenLane Configuration

openlane/soc_top/config.json { "DESIGN_NAME": "soc_top", "VERILOG_FILES": "dir::../../phase5_integration/rtl/*.sv", "CLOCK_PORT": "clk", "CLOCK_PERIOD": 15.0, "FP_CORE_UTIL": 40, "PL_TARGET_DENSITY": 0.45, "SYNTH_STRATEGY": "AREA 0" }

Run OpenLane cd openlane/ python -m openlane soc_top/config.json # This runs: Synthesis → Floorplan → Placement → CTS → Routing → DRC → LVS # Output directory: runs/RUN_YYYY_MM_DD_HHMMSS/ # Key files in outputs/: # final/gds/soc_top.gds ← the layout # reports/synthesis/ ← area report # reports/signoff/sta/ ← timing report (Fmax) # reports/signoff/power/ ← power report (mW)

## Numbers to Extract and Put on Resume

| Total cell area | From synthesis area report — e.g. '0.28 mm² on Sky130 130nm' |
| --- | --- |
| Fmax | From STA report — e.g. 'timing met at 66 MHz' |
| Dynamic power | From power report — e.g. '8.4 mW at 1.8V, 66 MHz' |
| Integration test | 'End-to-end: RISC-V firmware offloads 4×4 matrix multiply, verified correct' |
| Formal proofs | '4 SVA properties proved on arbiter (SymbiYosys)' |
| GDS screenshot | Save to docs/ — this is your 'chip photo' for the README |

## Phase 6 Completion Checklist

| ☐ | OpenLane run completes with zero DRC violations |
| --- | --- |
| ☐ | Area report: record total cell area in mm² |
| ☐ | Timing report: record Fmax in MHz |
| ☐ | Power report: record dynamic power in mW |
| ☐ | GDS screenshot saved to docs/soc_gds.png |
| ☐ | All numbers recorded in README.md results table |

| PHASE 7 Documentation + GitHub + Interview Prep | ⏱ Week 17-18 |
| --- | --- |

## README.md — What It Must Contain

| Title + badges | Project name, tool badges (SystemVerilog, cocotb, OpenLane, Sky130) |
| --- | --- |
| One-line summary | 'A complete RISC-V AI SoC built from scratch — synthesized on Sky130 130nm' |
| Architecture diagram | Block diagram showing all 5 modules and their connections — make in draw.io, export PNG |
| GDS screenshot | The layout image from OpenLane — your 'chip photo' |
| Results table | Area, Fmax, power, test coverage — in a markdown table |
| Phase breakdown | One paragraph per phase — what you built and why |
| How to run | Single commands: make simulate, make formal, make synth |

## The 30-Second Interview Pitch

Memorise This and Adapt It "I built a complete RISC-V AI SoC from scratch using open-source tools. It started with a bare 5-stage RV32I pipeline, then I added a write-back L1 cache, connected everything through an AXI4 crossbar, and attached a systolic array accelerator — the same architecture used in Google TPU. The RISC-V core can offload 4×4 matrix multiply to the accelerator via memory-mapped registers. I formally proved the arbiter with SymbiYosys, verified the full system with cocotb, and synthesized through OpenLane on Sky130: [AREA] mm² area, [FMAX] MHz, [POWER] mW."

## Final Resume Block

RISC-V AI SoC — Full-Stack RTL Design & Verification ¦ SystemVerilog · cocotb · SymbiYosys · Yosys · OpenLane · Sky130 Designed and verified a complete SoC from scratch across 5 incremental phases: Phase 1 — 5-stage RV32I pipeline: hazard detection, data forwarding, static branch prediction Phase 2 — Direct-mapped write-back L1 cache (4KB): 95% hit rate; verified against Python golden model Phase 3 — AXI4 3×3 crossbar + AXI-APB bridge: arbiter formally proved with SymbiYosys (4 SVA properties) Phase 4 — 4×4 systolic MAC array (weight-stationary): verified vs numpy, 100 random INT8 matrix multiplications Phase 5 — Full SoC integration: RISC-V firmware offloads matrix multiply to accelerator end-to-end Synthesized with OpenLane on Sky130 130nm: [X] mm² · [X] MHz · [X] mW dynamic power

# Master Timeline — Week by Week

| Weeks 1-2 | Phase 0: Install tools, set up GitHub repo, learn SystemVerilog basics on HDLBits |
| --- | --- |
| Week 2 | Phase 1 Start: Build ALU and register file, simulate in isolation |
| Weeks 3-4 | Phase 1: Build IF and ID stages, immediate generator, first pipeline registers |
| Weeks 4-5 | Phase 1: Build EX, MEM, WB stages |
| Weeks 5-6 | Phase 1: Add hazard unit and forwarding unit — the hard part |
| Week 7 | Phase 1: cocotb tests + Yosys synthesis. Phase 1 complete and committed. |
| Weeks 7-8 | Phase 2: Build cache FSM, tag array, data array |
| Week 9 | Phase 2: cocotb golden model tests + hit rate analysis. Phase 2 complete. |
| Weeks 9-10 | Phase 3: Build AXI4 arbiter and crossbar routing |
| Weeks 10-11 | Phase 3: Build AXI-APB bridge + CDC synchronisers |
| Week 12 | Phase 3: SymbiYosys formal proofs + cocotb interconnect tests. Phase 3 complete. |
| Weeks 12-13 | Phase 4: Build pe.sv + systolic_array.sv in isolation |
| Weeks 13-14 | Phase 4: Build AXI4-Lite slave wrapper + accel_top |
| Week 15 | Phase 4: cocotb numpy verification + connect to crossbar. Phase 4 complete. |
| Week 15-16 | Phase 5: Wire soc_top.sv + end-to-end firmware test. Phase 5 complete. |
| Weeks 16-17 | Phase 6: OpenLane synthesis — extract area, Fmax, power numbers |
| Weeks 17-18 | Phase 7: README, architecture diagram, interview prep. Ready to apply. |

Build it phase by phase. Commit everything. Own the story.